| Evolution Type |  |  | Scenario # | Burn # |  | Date |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Pre Burn Planning |  |  |  |  |  |  |  |
|  | PRE-BURN plans developed based on scenario and objectives established by Lead Instructor |  |  |  |  |  |  |
|  | Use Burn Building Command Sheet or White Board to document the following |  |  |  |  |  |  |
|  | - Location of Command Post |  |  |  |  |  |  |
|  | - Location of emergency escape routes, evacuation and assembly areas |  |  |  |  |  |  |
|  | - Location of ingress and egress routes for emergency vehicles |  |  |  |  |  |  |
|  | - Position of all apparatus |  |  |  |  |  |  |
|  | - Position of all hoses, including back-up lines |  |  |  |  |  |  |
|  | - Communication and radio plan for Instructors & Students |  |  |  |  |  |  |
|  | Separate water sources established for attack and back-up hose lines |  |  |  |  |  |  |
|  | Parking areas designated for apparatus staging, ambulance. |  |  |  |  |  |  |
|  | Periodic weather reports obtained & reviewed |  |  |  |  |  |  |
| Building Preparation |  |  |  |  |  |  |  |
|  | Safety Officer Checklist assigned & initiated |  |  |  |  |  |  |
|  | Computer & Thermocouples installed and operational |  |  |  |  |  |  |
|  | Adjacent Training evaluated for impact of Live Fire Training |  |  |  |  |  |  |
| Fire Sets Prepared |  |  |  |  |  |  |  |
|  | Crews briefed on size of sets for each Burn Area |  |  |  |  |  |  |
|  | Class A materials only - No Flammable or Combustible Liquids or contaminated materials |  |  |  |  |  |  |
|  | Fire Sets only built in Stainless Steel Lined Areas. No Fires outside of these areas |  |  |  |  |  |  |
|  | Ignition method & process discussed and understood |  |  |  |  |  |  |
|  | Propane Tank to be removed from building after each ignition |  |  |  |  |  |  |
|  | Burn Sequence Matrix chart |  |  |  |  |  |  |
|  | Burn Area | Max Fuel Load |  |  | Number of Sequential Burns | Cooling Time |  |
|  | 1st Floor South | 75 lb. (3 Pallets) / ¾ Bale |  |  | 2 | 30 Minutes |  |
|  | 1st Floor North | 100 lb. (4 Pallets) / 1 Bale |  |  | 2 | 30 Minutes |  |
|  | 2nd Floor South | 100 lb. (4 Pallets) / 1 Bale |  |  | 2 | 30 Minutes |  |
|  | Note: A single pallet used to keep the set off the floor with no hay is allowed. |  |  |  |  |  |  |
| Pre Burn Procedures |  |  |  |  |  |  |
|  | Instructor & Staff Assignments completed |  |  |  |  |  |
|  | Scenario's provided and reviewed with Students & Instructors |  |  |  |  |  |
|  | Instructor & Staff Walk Through completed |  |  |  |  |  |
|  | All Student Participants Briefed |  |  |  |  |  |
|  | All Participants Vitals Taken / Rehab Reviewed |  |  |  |  |  |
|  | Building Interior Walk Through |  |  |  |  |  |
|  | Crew and Instructor Introductions & Assignments |  |  |  |  |  |
|  | Crew Integrity and Rotations Explained |  |  |  |  |  |
|  | Safety Rules - PARS reinforced |  |  |  |  |  |
|  | Building Evacuation Procedures / Signal |  |  |  |  |  |
|  | Communications Method Reviewed |  |  |  |  |  |
|  | Scenario reviewed |  |  |  |  |  |
| Post Burn Procedures |  |  |  |  |  |  |
|  | All personnel accounted for |  |  |  |  |  |
|  | Document any injuries – Notify the Academy Director immediately if transported. |  |  |  |  |  |
|  | Remaining fire(s) overhauled, Burn rooms cleaned and remaining materials removed from Bldg. |  |  |  |  |  |
|  | Personal Decon and Decon of Contaminated PPE & SCBA's completed |  |  |  |  |  |
|  | Building inspected for hazards |  |  |  |  |  |
|  | Training critique conducted |  |  |  |  |  |
|  | All Equipment Picked Up and Returned to its proper location |  |  |  |  |  |
|  | All debris cleaned up &removed to proper locations with Tipper Buckets move away from Bldg. |  |  |  |  |  |
| Lead Instructor Accountability |  |  |  |  |  |  |
|  | Verify no injuries or illnesses sustained by participants and instructors |  |  |  |  |  |
|  | Documentation of any injuries incurred and treatment rendered |  |  |  |  |  |
|  | Documentation of unusual conditions or events |  |  |  |  |  |
|  | Documentation of changes or deterioration of Training Center Burn Building |  |  |  |  |  |
|  | Completes walk of building and grounds to verify completion of all tasks |  |  |  |  |  |
|  | Records & Reports completed |  |  |  |  |  |
|  | All documentation turned in |  |  |  |  |  |

| Pre Burn Comments |  |  |  |  |
| --- | --- | --- | --- | --- |
| Post Burn Comments |  |  |  |  |
| Prepared By |  |  | Date |  |
| Verification Acknowledgement |  |  |  |  |
| Lead Instructor |  |  | Date |  |
| Safety Officer |  |  | Date |  |
| Attachments Completed and Turned In |  |  |  |  |
|  | Burn Building Command Sheet |  |  |  |
|  | Safety Officer Checklist |  |  |  |
|  | Baseline Vitals / Rehab |  |  |  |
|  | Participant Agreement and Waiver |  |  |  |
|  | Live Fire Safety Briefing |  |  |  |
|  | Injury Reports (if required) |  |  |  |